BEACON

B E H A V I O U R - A W A R E   B A T T E R Y   M O N I T O R I N G

Calibration, Validation and Hardware Integration of a

Refusal-Driven Battery Health Monitoring System

Headline result. The prediction target this project was built around carries a maximum attainable R² of 0.044 — roughly 96% of its variance is measurement noise. Consequently the system reports what it can measure and refuses what it cannot support, and the refusals are the engineering deliverable.

Technical Project Report

Department of Computer and Communication Engineering

18 September 2026

# Contents

# Abstract

BEACON is a three-tier battery telemetry and health-monitoring system that ingests data from a CAN bus, a USB serial bench rig, or research datasets, converges all three on a single unified schema, and scores them through one shared pipeline into health, risk and remaining-useful-life estimates.

The system began as a rule-based scoring layer with hand-picked weights and thresholds, entirely unvalidated against real degradation data. This report documents the calibration effort undertaken against two public datasets — NASA Li-ion cycling data (34 cells) and CALCE PLN pouch cells — and reports four principal findings.

- The original heuristic health index shows no statistically significant relationship with measured capacity fade (Spearman ρ = −0.269, p = 0.124, n = 33).

- One behavioural signal, temperature exposure, shows a real, cohort-controlled, statistically significant relationship with fade — but does not transfer across ambient conditions, for a measurement reason identified in Section 7.

- The prediction target originally used, per-cycle capacity delta, has a signal fraction of 0.044. No method could have exceeded an R² of 0.044 against it. The central null result was therefore substantially an artefact of target definition.

- At the sample sizes standard battery datasets provide, a single leave-one-cohort-out estimate cannot distinguish between candidate methods: the estimator is approximately 2.7 times noisier than the effect it is used to rank. Six method-ranking claims were made during this work and all six were subsequently withdrawn.

The engineering response to these findings is a validation gate that refuses to emit metrics the data cannot support, returning a stated reason in place of a value. This report also documents a physical bench rig built around an INA219 current/voltage sensor and an ESP8266, which measures a real 18650 cell, and demonstrates the refusal mechanism operating on live hardware.

# 1. Introduction and Objectives

## 1.1 Problem statement

Battery management systems in electric vehicles and stationary storage estimate state of health (SOH) and remaining useful life (RUL) from operational telemetry. A recurring weakness in the applied literature is that such estimates are validated by random or k-fold cross-validation over pooled cycle data, which leaks information between training and test folds: cycles from the same cell, and often adjacent cycles, appear on both sides of the split.

This project set out to build a behaviour-aware monitoring system — one that scores a battery from how it has been used, not only from how old it is — and to validate it honestly.

## 1.2 Objectives and outcomes

| Objective | Outcome |
| --- | --- |
| O1 — Ingest telemetry from heterogeneous transports onto one schema | Met. CAN, USB serial and dataset replay converge on one unified schema and one scoring pipeline. |
| O2 — Derive behavioural features from usage telemetry | Met. C-rate, depth-of-discharge, thermal exposure and cycle segmentation implemented and tested. |
| O3 — Serve results through an API and dashboard | Met. FastAPI service, Express gateway, React client. |
| O4 — Demonstrate that behavioural scores predict capacity fade | Not met, and the reason is this project’s main result. See Sections 4–8. |
| O5 — Validate the ingest path on physical hardware | Met for voltage and current; temperature channel absent. See Section 9. |

On O4. A project that reported O4 as met would have had to ignore the evidence in Sections 4 and 6. This report treats not meeting it as the finding. The result that replaced it — that the standard validation protocol in this field cannot support the claims routinely made on it at these sample sizes — does not depend on batteries and is testable by anyone with two protocols and a held-out group.

# 2. System Architecture

The system is built in Python (FastAPI, pandas, scikit-learn), Node (Express), React (Vite) and C++ (Arduino firmware). It comprises approximately 27,500 lines across 33 test modules with a seven-job continuous-integration pipeline.

## 2.1 Data flow

CAN bus  ─┐

USB rig  ─┼─►  unified schema  ─►  feature layer  ─►  coverage gate  ─►  scoring  ─►  REST API  ─►  dashboard

datasets ─┘

The coverage gate sits between the feature layer and scoring. Every value reaching the dashboard has passed it, or arrives as a refusal carrying its reason.

## 2.2 Refusals as return values

Refusals are returned inside the result object rather than raised as exceptions. This is a deliberate architectural decision: an exception would become an HTTP 500 and lose the reason, whereas a refusal travels through the pipeline and reaches the user interface intact, where it is rendered in place of the value it replaces.

The governing rule, recorded as Architecture Decision Record 0004:

Any metric the input data cannot support is rendered as unavailable, with its reason, and is never replaced by a substituted or default value.

This has a practical consequence for anyone maintaining the system: a refusal in the output is not a defect. Making a number appear where the system has declined to produce one would defeat the system’s purpose.

## 2.3 Decision records and evidence pinning

Fourteen Architecture Decision Records under docs/adr/ document each significant decision and what forced it. Separately, tests/test_reported_numbers.py pins every figure quoted in the documentation to the artefact it was computed from, with two assertions per claim: that the number is still present in the prose, and that the artefact still yields it.

This exists because figures went stale silently during development. The SOH ceiling moved from 0.612 to 0.569 when one cell left the screening criteria, and the NASA benchmark table was superseded twice; each was caught only by a human noticing. Derived figures are recomputed inside the test from recorded recipes rather than read from a stored column, so a changed recipe fails the test with the recipe stated.

# 3. Datasets and Experimental Frame

## 3.1 Sources

| Dataset | Cells | Cohort axis | Role |
| --- | --- | --- | --- |
| NASA Li-ion cycling | 34 (33 after screening) | Ambient temperature, 9 protocols | Primary calibration and validation |
| CALCE PLN pouch cells | 19 | Depth of discharge, 8 cohorts, 2 cell families | Independent replication |

## 3.2 Cohort definition

A cohort is a group of cells sharing an operating protocol. In the NASA data the cohort axis is ambient temperature; in CALCE it is depth of discharge. The distinction between holding out a cell and holding out a cohort is the central methodological concern of this report and is developed in Section 5.

The cohort axis differs deliberately between the two datasets. A replication that reproduced the result on the same axis would establish only that the analysis is repeatable; reproducing it on a different axis establishes that the effect is a property of protocol shift rather than of temperature specifically.

# 4. The Target-Definition Problem

This section should be read before any result in this report. It determines what the other numbers mean.

## 4.1 Method

The routine benchmarks/targets.signal_to_noise decomposes each candidate target’s within-cell variance into monotone trend and residual, using isotonic regression against cycle index. A monotone fit assumes only that degradation does not reverse — the weakest assumption that still defines a trend. It was chosen deliberately over a polynomial, which would bend to absorb noise and overstate the recoverable signal.

The resulting trend fraction is a ceiling on attainable R², because nothing can predict the residual.

## 4.2 Results

| Target | Signal fraction (max attainable R²) | Cells | Rows |
| --- | --- | --- | --- |
| capacity_loss | 0.044 | 33 | 2,680 |
| soh / cumulative_fade | 0.569 | 31 | 2,585 |
| horizon_fade_10 | 0.389 | 31 | 2,268 |
| horizon_fade_20 | 0.639 | 25 | 1,933 |
| horizon_fade_50 | 0.967 | 17 | 1,246 |

An R² of 0.044 was the best score obtainable on the target against which this project’s central negative result was originally measured.

## 4.3 Mechanism

The mechanism is arithmetic rather than subtle, and will be familiar to anyone who has differenced noisy capacity measurements. A per-cycle capacity delta is the difference of two noisy measurements: it inherits both error terms while cancelling almost all of the underlying trend.

The empirical distribution confirms this directly. Mean 0.003 Ah, standard deviation 0.132 Ah, minimum −2.61 Ah. A cell does not recover 2.61 Ah of capacity in a single cycle; that value is noise, and it is in the labels.

## 4.4 Consequence for the project’s central claim

The original negative result — that no method beat a constant predictor — was real. Its scope was wrong.

- Defensible: these scores do not predict per-cycle capacity delta, which is mostly measurement noise.

- Not defensible: these scores do not predict degradation.

Against the cleaner soh target (ceiling 0.569), four candidate methods do clear the bar the original target rejected. The methods were never as uninformative as the first framing suggested; the target was unsuitable.

This also resolves an earlier puzzle. Lengthening the prediction horizon improved out-of-sample rank correlation but not R², originally attributed to the cohort intercept generalising worse as loss magnitude grows. The ceiling table supplies a simpler contributing explanation: horizon length and signal fraction rise together, so part of what was being measured was the target becoming cleaner.

# 5. Validation Protocol: LOBO versus LOCO

## 5.1 Definitions

- LOBO — leave-one-battery-out. One cell is held out; its operating protocol remains represented in the training set.

- LOCO — leave-one-cohort-out. An entire operating protocol is held out. Substantially harder, and rare in the applied literature.

The literature on this data offers a calibration point for how much protocol matters. Published work reports R² ≈ 0.91 on the same NASA data under cell-wise validation and describes it as optimistically biased; a 119% gap between 5-fold cross-validation and LOBO on NASA has also been reported. This project’s contribution is to take the next step and hold out the protocol.

## 5.2 The collapse

Ranking 33 cells by measured fade_rate_ah_per_cycle, Spearman ρ, permutation-tested with 5,000 resamples:

| Candidate | ρ | p | Distinct outputs |
| --- | --- | --- | --- |
| v1 rule-based health index | −0.269 | 0.124 | 6 of 33 |
| v2 fitted OLS, in-sample | 0.870 | 0.0002 | 33 |
| v2, LOBO-refit (unseen cell, known protocol) | 0.841 | 0.0002 | 33 |
| v2, LOCO-refit (unseen protocol) | −0.295 | 0.097 | 33 |

The in-sample row is inadmissible and is shown only to make the gap visible: the shipped v2 coefficients include a fitted intercept per cohort estimated on these same cells, and fade rate varies strongly by cohort. Refitting inside each fold is what makes the remaining rows meaningful.

ρ = 0.841 becomes ρ = −0.295 when the held-out axis changes from cell to protocol. Same model, same data, same features.

## 5.3 Model selection on LOBO selects the wrong model

A benchmark of twelve published methods, including a genuine XGBoost implementation and a genuine LSTM over ten consecutive cycles per cell, was evaluated under both protocols. LOBO rank carries essentially no information about LOCO rank: Spearman ρ = +0.084, p = 0.795, n = 12. Selecting a method by LOBO costs 0.309 R² relative to selecting by LOCO.

# 6. Benchmark Results

Target soh, 2,585 rows, 31 cells, 9 cohorts, ceiling 0.569. Sorted by LOCO R², the column that matters.

| Method | LOBO MAE | LOCO MAE | LOBO R² | LOCO R² | Δ |
| --- | --- | --- | --- | --- | --- |
| xgboost | 4.21% | 7.62% | 0.732 | 0.459 | −0.273 |
| age_linear | 8.54% | 10.71% | 0.483 | 0.406 | −0.076 |
| elasticnet | 7.61% | 10.31% | 0.609 | 0.355 | −0.254 |
| gpr_matern | 4.48% | 17.06% | 0.725 | 0.207 | −0.518 |
| random_forest | 4.90% | 10.67% | 0.773 | 0.150 | −0.623 |
| svr_rbf | 6.91% | 14.29% | 0.651 | 0.087 | −0.564 |
| age_quadratic | 6.62% | 8.31% | 0.336 | 0.034 | −0.301 |
| age_isotonic | 6.99% | 8.50% | 0.278 | 0.018 | −0.260 |
| train_mean | 11.17% | 11.92% | 0.000 | 0.000 | 0.000 |
| lstm | 5.62% | 17.36% | 0.594 | −0.185 | −0.778 |
| hist_gradient_boosting | 4.12% | 10.27% | 0.770 | −0.265 | −1.035 |
| mlp | 4.15% | 14.19% | 0.731 | −0.623 | −1.354 |

## 6.1 Reading the table

- Every method loses skill under protocol shift. The question is only how much.

- The most robust method is age_linear, a straight line on cycle count, with Δ = −0.076. It reaches LOCO R² = 0.406 against the best model’s 0.459.

- All behavioural and nonlinear modelling therefore buys approximately 0.05 R² over counting cycles.

- The three methods with the best LOBO R² — random_forest (0.773), hist_gradient_boosting (0.770), mlp (0.731) — rank 5th, 11th and 12th under LOCO. Two of them score below the constant baseline.

The LSTM and MLP results deserve comment given how commonly such architectures are reported as successful on this dataset. Both achieve competitive LOBO scores and both fall below a constant predictor under protocol shift. This is consistent with them learning the cohort-specific intercept structure rather than a transferable degradation relationship.

# 7. The Reversible Thermal Offset

Section 5 established the cross-cohort collapse statistically. This section gives it a measurement explanation, which is the part most likely to interest a reader with electrochemical background.

## 7.1 Observation

Apparent state of health measured at cycle index ≤ 20 — that is, before meaningful degradation is possible — varies systematically with ambient temperature:

| Ambient | Apparent SOH at cycle ≤ 20 |
| --- | --- |
| 4 °C | 0.918 |
| 44 °C | 0.999 |

An 8-point spread in apparent SOH exists before any cell has aged. This is a reversible thermal offset in the measurement, not degradation: at low temperature, increased internal resistance and reduced reaction kinetics curtail the discharge at the cutoff voltage, so less capacity is extracted from an undegraded cell.

## 7.2 Why it inverts the cross-cohort relationship

Within a fixed-ambient cohort the offset is a constant, so it shifts the intercept and leaves the within-cohort temperature-versus-fade relationship intact. That relationship is real, cohort-controlled and statistically significant.

Across cohorts the offset switches on, and it is anti-correlated with genuine thermal aging: hot cells show higher apparent capacity from the offset, while simultaneously degrading faster in truth. The two effects oppose, the offset dominates at low cycle counts, and the apparent sign of the relationship reverses.

Consequence for the feature layer. The current-based signals avg_stress and aggressive_discharge_count are significant and positive in room-temperature cohorts, and significant and negative in cold cohorts. A model fitted across cohorts without accounting for the offset is fitting a sign change.

This is a strong argument for capacity normalisation at a controlled reference temperature before any cross-protocol comparison — a step this project does not perform, and which Section 11 lists as the most valuable single piece of future work.

# 8. Estimator Variance and Withdrawn Claims

Five ranking claims had been made and withdrawn by the midpoint of this work. Rather than make a sixth, a designed experiment of 220 measurements was run to determine why they were failing to replicate.

## 8.1 The coverage hypothesis, refuted

The proposed explanation was that the LOBO-to-LOCO gap reflects cohort coverage — that more cohorts in training would close it. This was refuted: the one dataset showing a significant raw correlation lost it entirely once training-set size was regressed out. On the replication, raw ρ(cohorts, gap) = +0.126, p = 0.217, n = 98.

## 8.2 What the experiment did establish

The LOCO estimate has an interquartile range across cohort draws of 0.724 R², against a median difference between a nonlinear model and a straight line on cycle count of 0.351 R².

The measurement is approximately twice as noisy as the effect it is being used to rank. Every ranking claim failed to replicate because the instrument could not resolve the quantity.

## 8.3 Self-correction against a corrected target

That result was then itself re-examined against a corrected target derivation — same cells, same features, same two methods, same machinery, differing only in how the target was derived, restricted to the three coverage levels both runs draw fully (k = 3, 4, 5).

| Quantity | Cycle_Index target | Full-discharge target |
| --- | --- | --- |
| LOCO IQR of the gap | 0.892 | 0.296 |
| Median paired svr_rbf − age_linear | −0.336 | −0.110 |
| Draws where svr_rbf wins | 13.9% | 38.9% |
| Noise-to-effect ratio | 2.66 | 2.70 |

The IQR reduction is 0.596 R², bootstrap 95% confidence interval [0.335, 0.936] over 4,000 resamples; the corrected estimate is smaller in every resample. Three distinct conclusions follow, and they are not the same conclusion:

- The claim survives as a ratio. 2.70 against 2.66 — the estimator is several times noisier than the effect on both derivations. This is not an artefact of the target.

- The magnitudes were inflated approximately threefold. Quoting 0.724 or 0.351 as a property of leave-one-cohort-out in general overstates it by that factor; those figures describe that estimator on that frame.

- The mechanism does not survive. On the Cycle_Index target svr_rbf collapsed hardest; on the corrected target the ordering inverts. “The flexible method transfers worst” is the sixth withdrawn ranking claim.

## 8.4 Standing position

At the sample sizes standard battery datasets provide, a single leave-one-cohort-out estimate cannot distinguish between candidate methods and must be reported as an interval.

This is a methodological contribution independent of which model wins, and it is the result this project would offer for publication.

## 8.5 Threshold reachability of the risk score

A separate audit examined whether the rule-based risk score’s hand-chosen cut points are reachable by real data. Of a mean risk score of 52.3 points, 32.0 points — 61% — come from terms that are constant across the entire NASA fleet, because their thresholds sit outside the range the data occupies.

This explains mechanically why the heuristic showed no resolution between cells, and reframes the earlier null result as a calibration-design fault rather than a failure of the behavioural premise itself.

# 9. Hardware Integration

The analysis above is dataset work. This section documents the physical bench rig built to close the loop from instrument to dashboard, and is reported with a strict separation between what was measured and what was not.

## 9.1 Component selection and rationale

| Function | Selected | Rejected alternative and reason |
| --- | --- | --- |
| Current and bus voltage | INA219, I²C, 0.1 Ω 2 W shunt, high-side | ACS712 — analogue output inherits ADC nonlinearity, and its zero-current offset drifts with temperature, which is fatal for coulomb counting. |
| Terminal voltage | INA219 bus channel | Resistive divider to ADC — adds 1.4% resistor-tolerance error, drains the cell continuously, and the ESP8266 ADC is nonlinear near the rails. |
| Temperature | LM35DZ (analogue, fitted); DS18B20 preferred | NTC thermistor — a disconnected NTC floats to a plausible in-range value and is scored as a measurement. A disconnected DS18B20 reports −127 °C and is rejected by the schema. |
| Protection | Protected cell plus TP4056/DW01 module | None — the microcontroller is a monitor and must never be the protection device. |

The temperature selection criterion is worth stating explicitly because it is a systems argument rather than a component one: the part whose failure mode is detectable was preferred over the part with better nominal accuracy. The schema declares temperature valid over −40 to 150 °C, so a sensor whose failure produces a value inside that range is indistinguishable from a working one.

## 9.2 Shunt sizing

The INA219 shunt channel is ±320 mV full scale at 12 bits. With a 0.1 Ω shunt this gives a full-scale current of 3.2 A and a least significant bit of 100 µA. Discharge must therefore stay below 3.2 A; saturation is not a graceful failure, since a saturated reading is silently clipped rather than flagged. At 2 A the shunt dissipates 0.4 W, and its temperature coefficient makes a hot shunt read high, which makes current read low — a systematic capacity error in one direction.

## 9.3 Wire protocol

The firmware-to-host contract is beacon.telemetry.v1. Every line carries the sentinel BEACON1, a record type, a body, and an optional NMEA-style XOR checksum appended as *HH:

BEACON1 D {"t":0.138,"v":3.9880,"i":0.0000,"soc":100.00}*0F

- Checksum optional, enforced when present. A checksum verified only sometimes is worse than none, so a line carrying *HH must verify or the record is refused.

- Sentinel resync. The parser searches for the sentinel anywhere in the line rather than requiring it at position zero. This was added after a real capture failure: a post-reset board emits boot output, so the HELLO line arrived concatenated with reset noise and was discarded as noise.

- Capacity declared, not assumed. The HELLO line carries capacity_ah, bounded to 0.01–500 Ah. C-rate is current divided by rated capacity, so an incorrect capacity silently rescales every behavioural feature. If neither the rig nor the caller supplies it, scoring refuses rather than defaulting.

- Sensor complement declared. The rig probes its sensors at startup and declares in HELLO only the channels it can actually supply. A channel that disappears mid-capture is then a detectable fault rather than a silent gap.

## 9.4 Cell under test

| Property | Value |
| --- | --- |
| Cell | HONGLI ICR-18650-2200mAh, protected |
| Chemistry | ICR (LiCoO₂) |
| Nominal voltage | 3.7 V |
| Rated capacity | 2200 mAh (2.2 Ah), declared on the wire by the rig |
| Energy | 8.14 Wh |

The rated capacity is the manufacturer’s printed figure and has not been independently verified. It is reported as declared, not measured, and the HELLO line labels it as such.

## 9.5 Measured results

Capture of 83 accepted data records over 82.2 s at 1 Hz, cell at rest with no load connected:

| Channel | Mean | Minimum | Maximum | n | Status |
| --- | --- | --- | --- | --- | --- |
| Terminal voltage | 3.9871 V | 3.9840 V | 3.9920 V | 83 | Measured |
| Current, signed | −0.0000 A | −0.0004 A | 0.0004 A | 83 | Measured |
| Cell temperature | — | — | — | 0 | Not supplied |

### Instrument characterisation

- Every voltage reading is an exact multiple of 4 mV, which is the INA219 bus-channel least significant bit. The converter is resolving at its native resolution with no observable drift across the capture.

- The current channel noise floor at zero current is ±0.4 mA over n = 83, with 48 of 83 samples reading exactly zero. This is a measured instrument characteristic, not a datasheet figure.

- Frame acceptance was 83/83 (100%), with zero checksum failures and zero non-conforming lines.

## 9.6 What the hardware does not establish

Stage A — transport validated. An earlier capture of 151/151 accepted records validated framing, checksums, sentinel resync and time monotonicity on physical silicon. That firmware ran synthetic sensor stubs: the numbers were generated on the board and carried over a real wire. It validates the transport, not any measurement. Stage B — measurement, partially. Voltage and current are genuine instrument readings from a physical cell. Temperature is absent: the LM35 did not read, the rig declined to declare the channel, and the coverage gate consequently refused to produce a health index, RUL or state of health. Not established. No coulomb-counting accuracy claim is made, because no independent reference instrument was available to compare against. No charge-side behaviour was recorded. No capacity measurement was performed.

The refusal in Stage B is reproduced here verbatim, because it is the system behaving as designed rather than failing:

signal coverage: INCOMPLETE

missing: temperature_c

MISSING temperature_c -> needed by behaviour features

(high_temp_flag, temp_rolling_*)

This rig cannot drive the full feature pipeline.

The pipeline accepted every record, compared the delivered channels against those the feature layer requires, and declined to score — naming the missing channel and the specific features that depend on it. The voltage and current measurements are unaffected and are reported alongside the refusal.

## 9.7 Integration constraint encountered

The CH340 USB-to-serial bridge on the development board accepts only 9600 and 19200 baud; 38400 and above are refused at the driver level. This was diagnosed by probing the baud ladder rather than assumed. The firmware’s serial rate is a build-time override, so the board-specific value was applied without changing the repository default of 115200, which would otherwise have desynchronised the firmware and host for every other user.

# 10. Limitations and Threats to Validity

## 10.1 Statistical

- Sample size. 33 cells across 9 cohorts is small for cohort-level inference. The estimator-variance result in Section 8 is precisely a statement about this limitation, and it applies to this project’s own conclusions as much as to anyone else’s.

- Cohort imbalance. Three to four batteries per cohort is insufficient to identify a mixed-effects variance component, which is why a full LOBO cross-validation of the mixed model was deliberately not run: with an unidentified variance component the resulting number would not have been interpretable.

- Single-frame estimates. Any figure computed from one LOCO draw should be read as an interval, not a point. This is why no ranking claim is offered.

## 10.2 Measurement

- The reversible thermal offset of Section 7 is not corrected for. Cross-cohort results should be read with that confound present.

- Capacity is derived from the datasets’ own reported discharge capacity and inherits whatever measurement protocol each laboratory used.

## 10.3 Hardware

- No independent reference instrument. Coulomb-counting accuracy is unvalidated and no accuracy claim is made.

- The temperature channel is not functional. All temperature-dependent behavioural features are unexercised on hardware.

- Measurements were taken at rest. No discharge above the 0.5 A rest threshold was performed, so cycle segmentation is unexercised on live hardware.

- Single cell, single rig, single capture. No repeatability statistics across cells or sessions.

## 10.4 Scope

- The literature survey is incomplete and is deliberately left in its incomplete state rather than padded.

- Committed conformal-prediction artefacts did not reproduce exactly on re-run, with a fold-count discrepancy. This is recorded as open rather than resolved, and the pinned figures were not adjusted to match the new run.

# 11. Conclusions and Future Work

## 11.1 Conclusions

- The prediction target originally chosen, per-cycle capacity delta, is approximately 96% measurement noise, with a maximum attainable R² of 0.044. Target definition dominated the original negative result.

- Model ranking under leave-one-battery-out validation carries essentially no information about ranking under leave-one-cohort-out validation (ρ = +0.084, p = 0.795, n = 12), and selecting on the former costs 0.309 R².

- Across twelve published methods, a straight line on cycle count is the most robust under protocol shift. All behavioural and nonlinear modelling buys approximately 0.05 R² over it.

- A reversible thermal offset of approximately 8 SOH points exists in apparent capacity before any degradation, is constant within a cohort and anti-correlated with true thermal aging across cohorts, and mechanically explains the sign reversal observed in cross-cohort fitting.

- At these sample sizes a single leave-one-cohort-out estimate is roughly 2.7 times noisier than the effect it is used to rank. Six ranking claims were made during this work and all six were withdrawn.

- A physical rig measures terminal voltage and current on a real cell, and the validation gate correctly refuses to produce health, RUL or SOH when a required channel is absent.

## 11.2 Future work, in order of value

- Capacity normalisation to a controlled reference temperature before cross-protocol comparison. Section 7 identifies this as the single largest confound in the cross-cohort analysis.

- An independent capacity reference instrument, enabling a coulomb-counting accuracy claim and closing the principal hardware limitation.

- A dataset with 8–10 or more batteries per cohort, sufficient to identify the mixed-effects variance component that the present data cannot.

- Restoration of the temperature channel and a full charge–discharge cycle on hardware, exercising cycle segmentation and the temperature-dependent features end to end.

- Interval reporting as standard for all leave-one-cohort-out estimates, following Section 8.4.

# Appendix A: Reproduction

Every command below runs on a fresh clone with no hardware, no database, no API keys and no dataset download.

git clone https://github.com/Navvu-gityhub/Behavior-Aware-BMS.git

cd Behavior-Aware-BMS

pip install -r requirements-dev.txt

python main.py                      # end-to-end run

python scripts/run_serial_demo.py   # rig -> parser -> gate

python -m pytest tests/ -q          # 33 test modules

Hardware capture replay, reproducing the figures in Section 9.5:

python scripts/run_serial_demo.py --replay \\

data/interim/rig_stage_b_voltage_verified.txt

Firmware build for the bench rig, with the board-specific serial rate and real sensors enabled:

arduino-cli compile --fqbn esp8266:esp8266:nodemcuv2 \\

--build-property "compiler.cpp.extra_flags=\\

-DSERIAL_BAUD=9600 -DUSE_REAL_SENSORS=1" \\

firmware/beacon_rig

## Key artefacts

| Path | Contents |
| --- | --- |
| docs/final_report.md | Full analysis, all sections referenced above |
| docs/adr/ | Fourteen architecture decision records |
| docs/hardware_design.md | Sensor selection, shunt sizing, error budget, safety |
| docs/protocol_design.md | beacon.telemetry.v1 wire format |
| tests/test_reported_numbers.py | Prose-to-evidence pinning for every quoted figure |
| reports/metrics/ | Tracked result artefacts; live test inputs |
| data/interim/rig_stage_b_voltage_verified.txt | The hardware capture reported in Section 9.5 |